# ML MODEL EVALUATION REPORT

MACHINE LEARNING MODEL EVALUATION REPORT
Project: Predictive Maintenance System — Turbine Fault Detection
Team: Applied ML Engineering
Date: July 2, 2024

1. OBJECTIVE
Develop a binary classification model to predict turbine bearing failures 48 hours in advance
using sensor telemetry data, reducing unplanned downtime by at least 30%.

2. DATASET
- Source: Industrial IoT sensor logs from 42 turbines over 36 months
- Total records: 4.2 million time-series observations
- Features: 28 sensor channels (vibration, temperature, RPM, oil pressure, acoustic emission)
- Class distribution: 96.3% normal, 3.7% pre-failure (imbalanced)
- Train/Validation/Test split: 70% / 15% / 15% (time-based split to prevent leakage)

3. MODELS EVALUATED
  Model A: Gradient Boosting (XGBoost) with SMOTE oversampling
  Model B: LSTM Recurrent Neural Network (sequence length: 48 hours)
  Model C: Isolation Forest (anomaly detection baseline)

4. RESULTS

Model A — XGBoost + SMOTE:
    Precision: 0.87  |  Recall: 0.79  |  F1: 0.83  |  AUC-ROC: 0.94
    False Negative Rate: 21% (missed failures — critical metric)

Model B — LSTM:
    Precision: 0.84  |  Recall: 0.91  |  F1: 0.87  |  AUC-ROC: 0.96
    False Negative Rate: 9% (best recall — preferred for safety-critical use)

Model C — Isolation Forest (baseline):
    Precision: 0.61  |  Recall: 0.72  |  F1: 0.66  |  AUC-ROC: 0.81

5. SELECTED MODEL
Model B (LSTM) is selected for production deployment due to its superior recall (0.91)
and lowest false negative rate (9%). In a safety-critical context, missing a failure
is more costly than a false alarm.

6. FEATURE IMPORTANCE (XGBoost, for interpretability reference)
  1. Vibration RMS (Channel 3)     — 22.4%
  2. Bearing temperature delta     — 18.7%
  3. Oil pressure variance         — 14.2%
  4. Acoustic emission peak        — 11.9%
  5. RPM standard deviation        — 9.3%

7. DEPLOYMENT PLAN
- Serve LSTM model via FastAPI endpoint on GPU-enabled Kubernetes pod
- Inference latency target: <50ms per prediction
- Model retraining scheduled quarterly using rolling 6-month window
- Monitoring: track precision/recall drift via Evidently AI dashboard

8. LIMITATIONS
- Model performance may degrade for turbine models not represented in training data
- Requires minimum 6 hours of continuous sensor data for reliable prediction
- LSTM inference is not interpretable; XGBoost retained as explainability fallback